Purdue University

Grid Connected Single Phase Double Stage Inverter Control with PV Array, Boost Converter and MPPT Control

Project # 2

Majharul Hoque 11-10-2024

ECE 61016: Power Electronic Converters and Systems

Purdue University

Project Assignment #2

Name: Majharul Hoque

- Design a solar panel based on the specification given below:

Solar panel rating (single array)

Vmpp = 51.671 V

Impp = 6.06 A

Pmpp = 313.2 W

There are 2 arrays parallel connected, and 3 series connected providing 6 x Pmpp when the irradiance is at 1,000 W/m2.

- Use MATLAB Simulink to model the above solar panel and plot the IV curve when the irradiances are 1000, 800, 600, 400, and 200 W/m2.

- Design a boost converter has an input voltage of PV Vmpp and the required output voltage is 300 V for single-phase grid connection. The switching frequency is at 7 kHz. Design LC filter that meets:

  - Inductor ripple current < 15% of the output current

  - Output capacitor value > 5000 F

- Implement MPPT controller block with perturb and observe algorithm and validate the operation with different irradiances (1000 and 500 W/m2)

A general subsystem of the PV based Boost converter and use of MPPT system to keep output at the maximum level is shown below:

Figure 1: A general PV based Boost converter with MPPT algorithm implementation for Maximum Power Point.

A SIMULINK model of the specified solar panel or PV system has been shown below:

- The design is a general solar PV system. No commercially available PV system is used for the simulation:

Solar panel rating (single array)

Vmpp = 51.671 V

Impp = 6.06 A

Pmpp = 313.2 W

The below SIMULINK model is only for the PV system. The settings have been shown below :

Calculations for the Combined Array

- Cells per module (Ncell):

Each panel typically has 81 cells.

- Open Circuit Voltage (Voc):

The open-circuit voltage for one panel is 51.671 V. Since there are 3 panels in series, the total open-circuit voltage is: Voc_total=3×51.671=155.013 V.

- Short-Circuit Current (Isc):

The short-circuit current for one panel is 6.06 A. With two parallel arrays, the total short-circuit current is: Isc_total=2×6.06=12.12 A.

- Voltage at Maximum Power Point (Vmp):

The voltage at the maximum power point for one panel is 51.671 V. With three panels in series, the total maximum power point voltage is: Vmp_total=3×51.671=155.013 V

- Current at Maximum Power Point (Imp):

The current at the maximum power point for one panel is 6.06 A. With two parallel arrays, the total current at maximum power is: Imp_total=2×6.06=12.12 A

- Maximum Power (W):

The maximum power output for one panel is 313.2 W. With 6 panels (2 in parallel and 3 in series), the total maximum power is: Ptotal=6×313.2=1879.2

Figure 2: Voltage Vs. Power plot for the specified settings.

Figure 3: Voltage Vs. Power plot for the specified settings.

Now, the PV system has been connected to a Boose converter as shown in the model below. Here:

Input Voltage, Vin=3 X Vmpp=155.013V

Output Voltage, Vout=300V

Switching Frequency, fsw=7kHZ

Duty cycle, D=1- VinVout=0.483

Inductor ripple current, , ∆IL<15% of Iout=0.15 X 6.264A=0.9396A

Capacitor, C < 5000 µF

Inductor, L= Vin X Dfsw X ∆IL=~12 mH

A 5000Ω load resistor is placed at the output.

Figure 4: The PV system is connected to a Boost Converter with a MPPT controller block is also shown.

Figure 5: The Boost converter design in SIMULINK.

Figure 6: Maximum Power Point Tracking Controller block.

MPPT Controller Using Perturb and Observe:

The P&O algorithm adjusts the voltage to find the maximum power point (MPP). If increasing voltage results in increased power, the controller continues in that direction. If power decreases, it reverses the direction.

Figure 7: MPPT subsystem tracks the Output power and keeps tries to keep it at the maximum level.

Figure 8: Flowchart of the MPPT algorithm.

In the below images for difference irradiance levels the MPPT keeps the system tracking the maximum power point level. The first sets of Power, Voltage and current plots are for 1000 W/m2 irradiance level.

Irradiance level: 1000 W/m2

Figure 9: PV Output power Vs. Boost Converter Output power.

Figure 10: MPPT tracking in work, keeping the voltage around ~`155V level

Figure 11: Output Current of the PV system around ~12.5V

Irradiance level: 500 W/m2

Figure 12: AT 500 W/m2 irradiance level the output power went down but still the MPPT is keeping the system at Maximum Power Level.

Figure 13: Output Voltage around ~160V and Current of the PV system around ~12.5A

- Design a single-phase two-level inverter using MATLAB Simulink with below spec:

DC-link voltage: 300 V

Fundamental frequency: 60 Hz

Switching frequency: 10 kHz

Sampling frequency: 2 MHz

Output load: 5 ohm and 5 mH (use single-phase series RLC branch in element tab)

- Plot 0.017s of fundamental signal and triangular waveforms.

- Record and plot 0.1s of line-to-line, line-to-neutral, and phase current waveform at modulation index 1.

- Record and plot 0.1s of line-to-line, line-to-neutral, and phase current waveform at modulation index 0.5.

See the SIMULINK design of a single-phase two-level inverter below:

Figure 14: single-phase two-level inverter using MATLAB Simulink

Figure 15: The Inverter Circuit with VDC = 300V

Figure 16: Controller switch logic for the Double stage IGBT switches.

Parameters for Repeating Sequence (Triangular Carrier Wave)

- Time Values:

Since the switching frequency is 10 kHz, the period T of the triangular wave is:

T=110kHZ=0.0001 s or 100 µs

The triangular wave will go from 0 to its peak and back to 0 within this period. To define the time values,

set it to half of this period for rising and falling slopes:

Time values=[0,  T2,  T]=[0,   50×10-6,     100×10-6]

- Output Values:

The carrier wave's peak should match the DC-link voltage for modulation purposes. Since the DC-link voltage is 300V, the triangular wave can vary between 0 and 300 V. Therefore:

Output values=[0, 300, 0]

The LCL filter seen at the output of the inverter has been chosen because it has better characteristics compared to L and LC filter. Since the Inverter is designed for 10kHZ switching frequency , so the values of Capacitor and inductors suggested in the lecture note has been used.

Figure 17: The LCL filter design at the output of the inverter.

The Fundamental frequency (60Hz) and the triangular waveform have been shown below. Comparing these two waveforms the gate voltages or the switching signals of the IGBT network is produced.

Figure 18: 0.018s of the fundamental signal and triangular waveform.

The Output voltage and current waveforms are also shown below when the modulation indexes are 1 and 0.5 respectively.

Figure 19: The yellow curve is the 300V DC voltage,  The green signal is the periodic waveform at the Output of the inverter with ~150V peak, and the red curve is the in phase current waveform. The modulation index is set to 1.

Figure 20:The yellow curve is the 300V DC voltage,  The green signal is the periodic waveform at the Output of the inverter with ~150V peak, and the red curve is the in phase current waveform. The modulation index is set to 0.5.

Comparing Figures 19 and 20, the Output voltage is getting smoother for higher modulation index. But, as can be seen from Figure 19 the boost converter produced a periodic Sine waveform from a 300V DC voltage. The capacitors and inductors could be tuned better to improve smothering of the signals.

- Connect PV, boost converter, and single-phase inverter and produce:

  - 1 kW active power to grid at 800 W/m2 irradiance

  - 500 W active power with 500 VAR reactive power at 500 W/m2 irradiance

  - 1 kVAR reactive power at 500 W/m2 irradiance

The topology of the whole Double stage single phase Grid connected inverter with PV, Boost converter, MMPT control and PLL Vector control has been shown below:

Figure 21: Simplified diagram of the whole system.

Figure 22: A more robust and detailed version of the whole system.

Below an attempt was made to connect the whole system of PV, Boost converter and single-phase inverter:

Figure 23: The PV sub-system block and Boost Converter block.

Figure 24: The DC link capacitor connected at the output of the Boost converter.

Figure 25: The completed Single Phase double stage inverter connected to the Dc-Link capacitor

Figure 26: A completed version of Figure 16, Controller logic for the IGBT switches. Notice the Vref. How the Vref signal has been generated will be shown later.

Figure 27: The MPPT controller block and connected to it is the PWM signal generator for boost converter IGBT switch.

Figure 28: beta = Grid voltage and Alpha = 90 degree shifted version of alpha (PLL sub-system). Coswt = current reference for active power and sinwt = current reference for reactive power.

Figure 29: Single phase vector control to generate VRef signal. (PR Controller)

To determine the values of the gain modules in the Single-phase vector control or PR controller, Let’s

Take,

controller time constant, Tc=150 µS

Kp =L1Tc= 1mH150 µS=6.67

Let’s say, Kr =100

Then,

ω2Kr= (2*pi*60)2100=1421.22

Figure 30: Grid Voltage and current in Phase. Sending Active power.

Figure 31: sending around 1000 Watts when irradiance is 800 W/m2 and Grid Voltage and current are in phase.

Figure 32: Grid Voltage and current are out of Phase. Sending Re-Active power.

Figure 33: sending around 1000 VAR when irradiance is 500 W/m2 and Grid Voltage and current are out of phase.

Some concluding remarks, this is a first pass on a complicated electrical system as a young engineer. There are lots of limitations and improvements can be made by removing those. A grid synchronization subsystem has not been used. That’s also an essential part of this kind of system. But, overall, this was a good learning experience. The next steps should be:

- to try to smooth out the output signals better.

- Tuning the PI controller Kp and Ki parameters.

- Adding grid synchronization.

- Modify the PLL to make it easier to understand how to push active and re-active power better.

REFERENCE:

- Various YOUTUBE videos were used to plan and understand the system.

- Lecture notes -   Purdue University ECE 61600.

- Project simulation file:

- Project-2 Files  - Password: 0009988